# TLE Orbit Visibility Analysis

## 1. Introduction

This project presents a Python-based analysis framework for evaluating satellite visibility and ground-station coverage using publicly available Two-Line Element (TLE) orbital data. The objective of the analysis is to demonstrate how raw orbital elements can be transformed into operationally useful mission-analysis information such as satellite pass predictions, visibility statistics, and ground-track visualization.

Using a satellite database (sat_db_tle.txt), orbital propagation was performed with the SGP4 propagation model, which is the standard algorithm used for predicting satellite positions from TLE data. The analysis was conducted over the time window:

9 June 2025 00:00 UTC – 10 June 2025 00:00 UTC

for a ground station located at:

- Latitude: 48.123° N

- Longitude: 9.832° E

- Altitude: 250 m

The program processes TLE data and performs several key tasks commonly used in flight dynamics and mission analysis:

### Orbit Propagation

The software reads spacecraft TLEs and propagates their orbital states over the selected time interval using the SGP4 model. This allows satellite positions to be estimated throughout the analysis window.

### Ground Station Pass Detection

For a given ground station location, the program determines when a satellite becomes visible above the horizon. For each detected pass, the following parameters are computed:

- Pass start time

- Pass end time

- Pass duration

- Time of maximum elevation (culmination)

### Azimuth and Elevation Analysis

During each pass, the satellite's azimuth and elevation angles relative to the ground station are calculated. These quantities describe the satellite's apparent motion across the sky and allow generation of elevation-time and azimuth-time plots.

### Ground Track Visualization

The subsatellite point (latitude and longitude of the satellite projection onto Earth) is computed during orbit propagation. These positions are used to produce ground-track plots, showing the satellite trajectory across the Earth's surface.

### Multi-Satellite Analysis

The program can ingest a file containing multiple spacecraft TLEs and propagate each object individually, allowing analysis of constellation-level visibility and coverage.

### Visible Satellite Count

For the specified ground station and time window, the program computes the number of satellites simultaneously visible above a given elevation mask and produces a time history of visible satellite counts.

To remove shallow horizon passes that are typically unusable for communication or observation, a minimum elevation mask of 10° was applied.

# 2. Results

## 2.1 Pass Predictions

The satellite propagation algorithm generated a pass table (passes.csv) describing all satellite passes over the ground station during the selected time interval.

Two cases were analyzed.

### Case A — No Elevation Filter (min elevation = 0°)

Five satellite passes were detected during the 24-hour analysis period.

| Pass | Start UTC | End UTC | Duration (s) | Duration (min) | Culmination UTC |
| --- | --- | --- | --- | --- | --- |
| 1 | 2025-06-09T00:05:57 | 2025-06-09T00:13:47 | 469.7 | 7.83 | 2025-06-09T00:09:41 |
| 2 | 2025-06-09T01:42:27 | 2025-06-09T01:48:04 | 336.2 | 5.60 | 2025-06-09T01:45:11 |
| 3 | 2025-06-09T19:43:26 | 2025-06-09T19:47:59 | 273.1 | 4.55 | 2025-06-09T19:45:41 |
| 4 | 2025-06-09T21:16:45 | 2025-06-09T21:23:29 | 403.8 | 6.73 | 2025-06-09T21:20:02 |
| 5 | 2025-06-09T22:51:03 | 2025-06-09T22:58:07 | 423.4 | 7.06 | 2025-06-09T22:54:28 |

The detected passes lasted between 4.55 minutes and 7.83 minutes, which is typical for Low Earth Orbit (LEO)satellites orbiting at altitudes of approximately 500–800 km.

These short visibility windows arise because satellites move at orbital velocities of approximately 7.5 km/s, rapidly crossing the sky relative to the ground station.

### Case B — Elevation Filter Applied (min elevation = 10°)

Applying a 10° elevation mask removes shallow horizon passes and retains only the higher-quality portions of each pass.

| Pass | Start UTC | End UTC | Duration (s) | Duration (min) | Culmination UTC |
| --- | --- | --- | --- | --- | --- |
| 1 | 2025-06-09T00:08:20 | 2025-06-09T00:11:07 | 166.7 | 2.78 | 2025-06-09T00:09:41 |
| 2 | 2025-06-09T21:19:05 | 2025-06-09T21:21:00 | 114.8 | 1.91 | 2025-06-09T21:20:02 |
| 3 | 2025-06-09T22:53:15 | 2025-06-09T22:55:44 | 148.8 | 2.48 | 2025-06-09T22:54:28 |

After filtering, three passes remain, with durations between 1.9 and 2.8 minutes.

These shorter durations occur because only the central high-elevation portion of each pass is considered.

Figure 1. Elevation versus time for all satellite passes on 9 June 2025, observed from the ground station located at 48.123°N, 9.832°E with an altitude of 250 m.

The left panel shows results without applying an elevation cutoff, where five satellite passes are detected during the analysis window. These passes exhibit durations of approximately 4–8 minutes, which is typical for satellites in Low Earth Orbit (LEO).

The right panel shows results after applying a minimum elevation mask of 10°, which removes shallow horizon passes that are generally unsuitable for communication or observation. Under this constraint, only three passes remain, with durations reduced to approximately 2–3 minutes, since only the higher-elevation portion of each pass is considered.

The comparison demonstrates how applying an elevation mask reduces the number of usable passes and shortens their effective duration, which is an important consideration in practical ground-station operations.

Figure 2. Azimuth versus time for all satellite passes on 9 June 2025 observed at the ground station (48.123°N, 9.832°E, altitude 250 m).
Left: passes detected without an elevation cutoff (5 passes, azimuth ranging approximately 130°–260°).
Right: passes detected with a 10° elevation cutoff, resulting in three visible passes.

The azimuth tracks correspond to the same events seen in the elevation plots, showing the satellite moving from southeast to southwest during each visible pass. Applying the elevation filter reduces the number of plotted arcs but does not alter the overall azimuth geometry.

Figure 3. Ground track of the satellite between 2025-06-09 00:00 UTC and 2025-06-10 00:00 UTC.
The track consists of coloured arcs representing consecutive orbital revolutions projected onto Earth's surface. The characteristic wave-like pattern is typical for a Low Earth Orbit satellite.

The starting point (pink dot), ending point (grey square), and the ground station (yellow triangle) at latitude 48.123°N and longitude 9.832°E are annotated.

The ground track is confined to approximately ±42° latitude, while the westward shift between successive passes results from Earth’s rotation beneath the orbit.

## 2.2 Satellite Visibility Over Time

Using the full satellite database (sat_db_tle.txt), the number of satellites simultaneously visible from the ground station was calculated.

Figure 4. Number of visible satellites over time for a ground station located at 48.123°N, 9.832°E with a 10° elevation mask and 60-second time step.

The plot shows that between approximately 140 and 220 satellites are simultaneously visible during the analysis window (9–11 June 2025).

These fluctuations arise from variations in orbital geometry, satellite inclination distributions, and the temporal phasing of large LEO constellations.

The results indicate that approximately 140 to 220 satellites were visible above the 10° elevation mask during the analysis period.

This high number reflects the presence of modern dense LEO satellite constellations, particularly broadband communication systems such as Starlink, along with other operational satellites contained within the TLE database.

The number of visible satellites fluctuates over time due to several orbital dynamics factors:

- orbital plane spacing

- inclination distribution

- satellite phasing

- Earth’s rotation relative to orbital planes

Peaks in the visibility plot occur when multiple orbital planes pass over the ground station simultaneously, while dips occur when fewer satellites occupy favorable geometric positions.

# 3. Discussion

The results demonstrate how satellite visibility is strongly influenced by orbital geometry, ground station location, and constellation density.

The pass prediction analysis confirms that individual LEO satellites typically produce short visibility windows lasting approximately 5–8 minutes, consistent with their high orbital velocities. When a minimum elevation mask is applied, only the central portions of the pass remain visible, reducing effective pass durations to roughly 2 minutes.

The constellation visibility analysis reveals that the modern Low Earth Orbit environment is extremely crowded. The observation that 140–220 satellites can be visible simultaneously highlights the rapid expansion of large commercial constellations.

Temporal variations in satellite visibility arise from the combined effects of:

- orbital plane distribution

- inclination diversity

- satellite phasing

- Earth's rotation relative to the orbital planes

Ground-track visualizations further illustrate how satellites sweep across Earth's surface in repeating patterns as the planet rotates beneath their orbital planes.

A comparison between satellites with different orbital inclinations also demonstrates how inclination affects ground coverage. Mid-inclination satellites, such as the ISS, remain confined to limited latitude bands, whereas near-polar satellites, such as NOAA-19, generate near north-south ground tracks and provide broader global coverage with frequent high-latitude revisits.

Overall, the analysis confirms that TLE-based propagation combined with pass prediction and visibility analysis provides an effective approach for evaluating ground station coverage and satellite constellation density.

# 4. Conclusion

This study demonstrated how publicly available TLE orbital data can be used to analyze satellite visibility, predict ground-station passes, and evaluate constellation density using standard astrodynamics techniques.

The results show that individual LEO satellites produce short visibility windows of several minutes, while modern satellite constellations ensure that large numbers of satellites are simultaneously visible from a single ground location.

Such analyses are essential for ground station scheduling, communication planning, and understanding the rapidly increasing density of spacecraft in Low Earth Orbit.

## Appendix C- Multiple TLE Analysis -Example

Running the case for ISS (ZARYA) and NOAA-19

| ISS (ZARYA) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 25544U 98067A | 25274.51094907 | 0.00014338 | 00000-0 | 25920-3 | 0 | 9990 |
| 2 25544 51.6437 | 353.0325 0005282 | 64.4153 | 61.2595 | 15.4977855 | 48 | 127 |
| NOAA 19 |  |  |  |  |  |  |
| 1 33591U 09005A | 25274.21375483 | 0.00000079 | 00000-0 | 72657-4 | 0 | 9998 |
| 2 33591 99.1967 | 289.2420 0014225 | 64.2102 | 296.0701 | 14.1250898 | 579 | 9354 |

1. Pass Predictions

ISS (ZARYA)

| sat | pass | start_utc | end_utc | duration_s | duration_min | culmination_utc |
| --- | --- | --- | --- | --- | --- | --- |
| ISS_ZARYA | 1 | 2025-06-09T01:18:38.791317Z | 2025-06-09T01:25:14.621960Z | 395.8 | 6.6 | 2025-06-09T01:21:56.475942Z |
| ISS_ZARYA | 2 | 2025-06-09T02:55:46.081697Z | 2025-06-09T03:02:37.100108Z | 411.0 | 6.85 | 2025-06-09T02:59:11.692411Z |
| ISS_ZARYA | 3 | 2025-06-09T04:33:00.495837Z | 2025-06-09T04:38:52.540759Z | 352.0 | 5.87 | 2025-06-09T04:35:56.644268Z |
| ISS_ZARYA | 4 | 2025-06-09T21:20:41.396620Z | 2025-06-09T21:25:13.861648Z | 272.5 | 4.54 | 2025-06-09T21:22:57.235071Z |
| ISS_ZARYA | 5 | 2025-06-09T22:56:05.905301Z | 2025-06-09T23:02:55.019275Z | 409.1 | 6.82 | 2025-06-09T22:59:29.663680Z |

- Five passes  above the ground station were recorded within the 24-hour window.

- Pass durations ranged between 4.5 and 6.8 minutes, typical for ISS overpasses.

- The maximum elevations were very high, nearly reaching 70–80°, meaning the ISS was passing nearly overhead.

NOAA-19

| sat | pass | start_utc | end_utc | duration_s | duration_min | culmination_utc |
| --- | --- | --- | --- | --- | --- | --- |
| NOAA_19 | 1 | 2025-06-09T04:36:33.319305Z | 2025-06-09T04:42:45.204635Z | 371.9 | 6.2 | 2025-06-09T04:39:39.442214Z |
| NOAA_19 | 2 | 2025-06-09T06:15:29.650735Z | 2025-06-09T06:26:35.402747Z | 665.8 | 11.1 | 2025-06-09T06:21:02.885701Z |
| NOAA_19 | 3 | 2025-06-09T07:57:29.208007Z | 2025-06-09T08:04:26.751266Z | 417.5 | 6.96 | 2025-06-09T08:00:57.875191Z |
| NOAA_19 | 4 | 2025-06-09T16:05:32.364764Z | 2025-06-09T16:15:40.171566Z | 607.8 | 10.13 | 2025-06-09T16:10:35.129084Z |
| NOAA_19 | 5 | 2025-06-09T17:46:22.938504Z | 2025-06-09T17:56:16.111548Z | 593.2 | 9.89 | 2025-06-09T17:51:17.888342Z |

- Five passes were also given in the same window.

- Pass durations varied more widely, ranging between 6 and 11 minutes, which was reflective of  its sun-synchronous polar orbit.

- Maximum elevations reached nearly 90° (direct overhead), confirming strong visibility.

2. Elevation vs Time Plots

Figure 5: Shows elevation against time for NOAA-19 (left) and ISS (ZARYA) (right) passes over the ground station (48.123°N, 9.832°E, 250 m) during 9 June 2025. NOAA-19 generated five passes attaining maximum elevations of nearly 90° and durations up to 11 minutes as expected for a polar sun-synchronous orbit. Likewise, the ISS ZARYA generated five passes, with maximum elevations of  ~80° and durations from 4.5 to 6.8 minutes, as is to be expected from a mid-inclination orbit. The comparison clearly brings out the orbital geometry dependence of pass frequency, highest elevation, and visibility duration.

- ISS: More frequent passes distributed over different times, with some passes nearly overhead.

- NOAA-19: Passes are more spread out during the course of the day with very steep elevation peaks.

- The elevation plots explain the coverage as a function of orbital inclination: While ISS at ~51.6° inclination delivers mid-latitude arcs, while NOAA-19 (~99° inclination) delivers near-polar coverage and longer visible arcs.

3. Azimuth vs Time Plots

Figure 6: Azimuth against time ) on 9 June 2025 for NOAA-19 (left) and ISS ZARYA (right) passes above the ground station (48.123°N, 9.832°E, 250 m. NOAA-19 pass shows strong north–south azimuth sweeps consistent with almost polar orbit, whereas those of  ISS ZARYA are mid-latitude azimuth sweeps with its 51.6° inclination. Each curve corresponds to a distinct pass and thus corresponds to an elevation plots.

- ISS: the wide range of Azimuth sweeps by the ISS as it passes arise from low horizons to a low end setting in almost opposite directions of azimuth, consistent with orientation of the inclined orbit.

- NOAA-19: as with a nearly polar pass, tracks show strong N–S azimuth sweeps.

4. Ground Track Visualizations

Figure 7 : Ground tracks of NOAA-19 (a) and ISS ZARYA (bottom) from 2025-06-09 00:00 UTC to 2025-06-10 00:00 UTC. NOAA-19 covers near pole-to-pole, hence the high-elevation opportunities, while ISS ZARYA should stay within ±51° in latitude, offering frequent but mid-inclination coverage. Start (pink dot), end (grey square), and the ground station (yellow triangle) are annotated for reference.

- ISS: The ground track gives very dense coverage between ±51° latitude, parallel its orbital inclination. The ground station (48.1°N) lies near the edge of this band, giving it several strong passes.

- NOAA-19: The track covers near  pole-to-pole movements because of the ~99° sun-synchronous orbit, ensuring that it gets to have near-zenith passes over the station multiple times a day.

Summary

By consuming multiple TLEs in a single run:

- The program has now been designed to operate on various satellites simultaneously, storing results (passes, plots, ground tracks) per satellite.

- Some clear results show the contrast between a mid-inclination orbit (ISS) and a polar orbit (NOAA-19): ISS provides short passes frequently at varying elevation angles, while NOAA-19 delivers high-elevation, polar-spanning passes of longer duration.

- This establishes a case for scalability of the pipeline for constellation or multi-mission analysis, where coverage of different satellites could be compared side-by-side.

## Appendix A- Multiple TLE Analysis -Example

Running the case for ISS (ZARYA) and NOAA-19

| ISS (ZARYA) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 25544U 98067A | 25274.51094907 | 0.00014338 | 00000-0 | 25920-3 | 0 | 9990 |
| 2 25544 51.6437 | 353.0325 0005282 | 64.4153 | 61.2595 | 15.4977855 | 48 | 127 |
| NOAA 19 |  |  |  |  |  |  |
| 1 33591U 09005A | 25274.21375483 | 0.00000079 | 00000-0 | 72657-4 | 0 | 9998 |
| 2 33591 99.1967 | 289.2420 0014225 | 64.2102 | 296.0701 | 14.1250898 | 579 | 9354 |

1. Pass Predictions

ISS (ZARYA)

| sat | pass | start_utc | end_utc | duration_s | duration_min | culmination_utc |
| --- | --- | --- | --- | --- | --- | --- |
| ISS_ZARYA | 1 | 2025-06-09T01:18:38.791317Z | 2025-06-09T01:25:14.621960Z | 395.8 | 6.6 | 2025-06-09T01:21:56.475942Z |
| ISS_ZARYA | 2 | 2025-06-09T02:55:46.081697Z | 2025-06-09T03:02:37.100108Z | 411.0 | 6.85 | 2025-06-09T02:59:11.692411Z |
| ISS_ZARYA | 3 | 2025-06-09T04:33:00.495837Z | 2025-06-09T04:38:52.540759Z | 352.0 | 5.87 | 2025-06-09T04:35:56.644268Z |
| ISS_ZARYA | 4 | 2025-06-09T21:20:41.396620Z | 2025-06-09T21:25:13.861648Z | 272.5 | 4.54 | 2025-06-09T21:22:57.235071Z |
| ISS_ZARYA | 5 | 2025-06-09T22:56:05.905301Z | 2025-06-09T23:02:55.019275Z | 409.1 | 6.82 | 2025-06-09T22:59:29.663680Z |

- Five passes  above the ground station were recorded within the 24-hour window.

- Pass durations ranged between 4.5 and 6.8 minutes, typical for ISS overpasses.

- The maximum elevations were very high, nearly reaching 70–80°, meaning the ISS was passing nearly overhead.

NOAA-19

| sat | pass | start_utc | end_utc | duration_s | duration_min | culmination_utc |
| --- | --- | --- | --- | --- | --- | --- |
| NOAA_19 | 1 | 2025-06-09T04:36:33.319305Z | 2025-06-09T04:42:45.204635Z | 371.9 | 6.2 | 2025-06-09T04:39:39.442214Z |
| NOAA_19 | 2 | 2025-06-09T06:15:29.650735Z | 2025-06-09T06:26:35.402747Z | 665.8 | 11.1 | 2025-06-09T06:21:02.885701Z |
| NOAA_19 | 3 | 2025-06-09T07:57:29.208007Z | 2025-06-09T08:04:26.751266Z | 417.5 | 6.96 | 2025-06-09T08:00:57.875191Z |
| NOAA_19 | 4 | 2025-06-09T16:05:32.364764Z | 2025-06-09T16:15:40.171566Z | 607.8 | 10.13 | 2025-06-09T16:10:35.129084Z |
| NOAA_19 | 5 | 2025-06-09T17:46:22.938504Z | 2025-06-09T17:56:16.111548Z | 593.2 | 9.89 | 2025-06-09T17:51:17.888342Z |

- Five passes were also given in the same window.

- Pass durations varied more widely, ranging between 6 and 11 minutes, which was reflective of  its sun-synchronous polar orbit.

- Maximum elevations reached nearly 90° (direct overhead), confirming strong visibility.

2. Elevation vs Time Plots

Figure 5: Shows elevation against time for NOAA-19 (left) and ISS (ZARYA) (right) passes over the ground station (48.123°N, 9.832°E, 250 m) during 9 June 2025. NOAA-19 generated five passes attaining maximum elevations of nearly 90° and durations up to 11 minutes as expected for a polar sun-synchronous orbit. Likewise, the ISS ZARYA generated five passes, with maximum elevations of  ~80° and durations from 4.5 to 6.8 minutes, as is to be expected from a mid-inclination orbit. The comparison clearly brings out the orbital geometry dependence of pass frequency, highest elevation, and visibility duration.

- ISS: More frequent passes distributed over different times, with some passes nearly overhead.

- NOAA-19: Passes are more spread out during the course of the day with very steep elevation peaks.

- The elevation plots explain the coverage as a function of orbital inclination: While ISS at ~51.6° inclination delivers mid-latitude arcs, while NOAA-19 (~99° inclination) delivers near-polar coverage and longer visible arcs.

3. Azimuth vs Time Plots

Figure 6: Azimuth against time ) on 9 June 2025 for NOAA-19 (left) and ISS ZARYA (right) passes above the ground station (48.123°N, 9.832°E, 250 m. NOAA-19 pass shows strong north–south azimuth sweeps consistent with almost polar orbit, whereas those of  ISS ZARYA are mid-latitude azimuth sweeps with its 51.6° inclination. Each curve corresponds to a distinct pass and thus corresponds to an elevation plots.

- ISS: the wide range of Azimuth sweeps by the ISS as it passes arise from low horizons to a low end setting in almost opposite directions of azimuth, consistent with orientation of the inclined orbit.

- NOAA-19: as with a nearly polar pass, tracks show strong N–S azimuth sweeps.

4. Ground Track Visualizations

Figure 7 : Ground tracks of NOAA-19 (a) and ISS ZARYA (bottom) from 2025-06-09 00:00 UTC to 2025-06-10 00:00 UTC. NOAA-19 covers near pole-to-pole, hence the high-elevation opportunities, while ISS ZARYA should stay within ±51° in latitude, offering frequent but mid-inclination coverage. Start (pink dot), end (grey square), and the ground station (yellow triangle) are annotated for reference.

- ISS: The ground track gives very dense coverage between ±51° latitude, parallel its orbital inclination. The ground station (48.1°N) lies near the edge of this band, giving it several strong passes.

- NOAA-19: The track covers near  pole-to-pole movements because of the ~99° sun-synchronous orbit, ensuring that it gets to have near-zenith passes over the station multiple times a day.

Summary

By consuming multiple TLEs in a single run:

- The program has now been designed to operate on various satellites simultaneously, storing results (passes, plots, ground tracks) per satellite.

- Some clear results show the contrast between a mid-inclination orbit (ISS) and a polar orbit (NOAA-19): ISS provides short passes frequently at varying elevation angles, while NOAA-19 delivers high-elevation, polar-spanning passes of longer duration.

- This establishes a case for scalability of the pipeline for constellation or multi-mission analysis, where coverage of different satellites could be compared side-by-side.